# Branch Predictor Implementation

Assignment 1 | Pentium M–inspired hybrid direction predictor

# 1 Introduction

This assignment implements a hybrid branch direction predictor that combines a bimodal table with a tagged global predictor. The bimodal table learns the tendency of a branch address, while the global table distinguishes recent branch-outcome histories. A matching global entry supplies the final prediction; otherwise, the bimodal table supplies it.

The C++ implementation completes pm_predictor in cbp2-infrastructure-v2/src/my_predictor.h and uses the supplied CBP2 trace driver. Evaluation covers all supplied benchmarks and measures conditional direction mispredictions per thousand executed instructions (MPKI).

# 2 Design and Code Changes

The design follows the assignment diagram and uses its permitted two-way global predictor. Both predictor components use saturating counters: states 0 and 1 predict not taken; states 2 and 3 predict taken.

| Component | Implemented configuration |
| --- | --- |
| Bimodal table | 4,096 entries; one 2-bit counter per entry; index = PC[11:0]. |
| Global table | 512 sets × 2 ways; each entry has a valid bit, a 6-bit tag, and a 2-bit counter. |
| Global history | 15 bits containing the most recent conditional outcomes; taken = 1, not taken = 0. |
| Global lookup | H = (PC XOR GHR) AND 0x7FFF; set = H[14:6]; tag = H[5:0]. |
| Replacement | Use the first invalid way; otherwise replace the least recently used way. |
| Initial state | GHR = 0; all counters = 1 (weakly not taken); all global entries invalid. |
| Training | Train bimodal on every conditional branch; train the matching or newly allocated global entry. |
| Model scope | Direction prediction only; the assignment permits omitting loop and target predictors and the PIR-specific hash. |

PC denotes the trace’s byte address. The 15-bit XOR directly supplies the nine set bits and six tag bits; no word-address shift is applied. The counters are stored in C++ bytes but are constrained to the four states of a 2-bit counter.

## 2.1 Predictor State and Initialization

The initial class has no prediction tables or history. The modified class adds the tables, per-set replacement state, and prediction-time metadata. Keeping the selected indexes and tag in pm_update ensures that update() trains the entries used by the preceding prediction.

### 2.1.1 Initial Version

my_predictor.h | pm_update and pm_predictor constructor | lines 57–67

class pm_update : public branch_update {
public:
        unsigned int index;
};

        class pm_predictor : public branch_predictor {
public:
        pm_update u;

                pm_predictor (void) {
        }

### 2.1.2 Modified Version

my_predictor.h | pm_update | lines 61–68

class pm_update : public branch_update {
public:
        unsigned int index;      // bimodal index at prediction time
        unsigned int set_index;  // global set at prediction time
        unsigned int tag;
        int way;                 // -1 for a global miss
        bool conditional;
};

my_predictor.h | pm_predictor configuration and state | lines 72–86

enum {
    BIMODAL_BITS = 12, SET_BITS = 9, TAG_BITS = 6,
    HISTORY_LENGTH = SET_BITS + TAG_BITS, WAYS = 2
};
struct global_entry {
    unsigned char tag;
    unsigned char counter;
    bool valid;
};

        pm_update u;
unsigned int history;
unsigned char bimodal[1 << BIMODAL_BITS];
global_entry global[1 << SET_BITS][WAYS];
unsigned char lru[1 << SET_BITS]; // next victim when both ways are valid

my_predictor.h | pm_predictor constructor | lines 88–98

pm_predictor (void) : history(0) {
    std::memset(bimodal, 1, sizeof(bimodal)); // weakly not taken
    std::memset(lru, 0, sizeof(lru));
    for (unsigned int set = 0; set < (1u << SET_BITS); ++set) {
        for (int way = 0; way < WAYS; ++way) {
            global[set][way].tag = 0;
            global[set][way].counter = 1;
            global[set][way].valid = false;
        }
    }
}

## 2.2 Prediction and Global-Hit Selection

The initial predict() always returns taken. The implementation indexes the bimodal table with the low 12 address bits and computes the global set and tag from the address/history XOR. It searches both ways and accepts only a valid matching tag. A hit selects the global counter; a miss selects the bimodal counter.

### 2.2.1 Initial Version

my_predictor.h | pm_predictor::predict | lines 69–78

branch_update *predict (branch_info & b) {

            // predict branch outcome
    u.direction_prediction (true);

            // predict branch target address
    u.target_prediction (0);

            return &u;
}

### 2.2.2 Modified Version

my_predictor.h | pm_predictor::predict | lines 100–123

branch_update *predict (branch_info & b) override {
    u.conditional = (b.br_flags & BR_CONDITIONAL) != 0;
    u.way = -1;
    u.direction_prediction (true);
    u.target_prediction (0); // target prediction is outside this assignment
    if (!u.conditional) return &u;

            u.index = b.address & ((1u << BIMODAL_BITS) - 1);
    const unsigned int hash = (b.address ^ history)
                              & ((1u << HISTORY_LENGTH) - 1);
    u.set_index = hash >> TAG_BITS;
    u.tag = hash & ((1u << TAG_BITS) - 1);
    for (int way = 0; way < WAYS; ++way) {
        const global_entry &entry = global[u.set_index][way];
        if (entry.valid && entry.tag == u.tag) {
            u.way = way;
            break;
        }
    }
    const unsigned char counter = u.way >= 0
        ? global[u.set_index][u.way].counter : bimodal[u.index];
    u.direction_prediction(counter >= 2);
    return &u;
}

Unconditional branches return taken and do not access the prediction tables. The target output remains zero because branch target prediction is outside the required model. Global misses are recorded as way = −1 for subsequent allocation.

## 2.3 Counter Training, Allocation, and History

The initial update() does nothing. The modified version trains bimodal on every conditional outcome. On a global miss, it first chooses an invalid way and otherwise uses the LRU victim. The selected slot keeps its previous counter and trains it with the actual outcome, following the worked updates in Appendix B. On a hit, it trains the matching entry.

### 2.3.1 Initial Version

my_predictor.h | pm_predictor::update | lines 80–82

void update (branch_update *u, bool taken, unsigned int target) {

        }

### 2.3.2 Modified Version

my_predictor.h | pm_predictor::train | lines 125–131

static void train(unsigned char &counter, bool taken) {
    if (taken) {
        if (counter < 3) ++counter;
    } else {
        if (counter > 0) --counter;
    }
}

my_predictor.h | pm_predictor::update | lines 133–157

void update (branch_update *update, bool taken, unsigned int) override {
    pm_update &prediction = *static_cast<pm_update *>(update);
    if (!prediction.conditional) return;

            train(bimodal[prediction.index], taken);
    int way = prediction.way;
    if (way < 0) {
        way = lru[prediction.set_index];
        for (int candidate = 0; candidate < WAYS; ++candidate) {
            if (!global[prediction.set_index][candidate].valid) {
                way = candidate;
                break;
            }
        }
        global_entry &entry = global[prediction.set_index][way];
        entry.valid = true;
        entry.tag = static_cast<unsigned char>(prediction.tag);
        // Appendix B trains the selected slot's existing counter,
        // including when that slot is allocated to a different tag.
    }
    train(global[prediction.set_index][way].counter, taken);
    lru[prediction.set_index] = static_cast<unsigned char>(1 - way);
    history = ((history << 1) | static_cast<unsigned int>(taken))
              & ((1u << HISTORY_LENGTH) - 1);
}

Counters saturate at 0 and 3. After either a hit or allocation, the other way becomes the next LRU victim. History shifts left, appends the actual conditional outcome, and retains 15 bits. Nonconditional branches leave counters, tags, replacement state, and history unchanged.

## 2.4 Direction-Misprediction Measurement

The driver keeps the supplied MPKI definition and counts direction errors before updating the predictor. It also records the exact number of conditional branches and direction misses, allowing the reported totals and average to be checked without relying on rounded MPKI output.

### 2.4.1 Initial Version

predict.cc | main: counters | lines 36–38

long long int 
    tmiss = 0,  // number of target mispredictions
    dmiss = 0;  // number of direction mispredictions

predict.cc | main: MPKI output | lines 81–81

printf ("%0.3f MPKI\n", 1000.0 * (dmiss / 1e8));

### 2.4.2 Modified Version

predict.cc | main: counters | lines 36–36

long long int conditional = 0, dmiss = 0;

predict.cc | main: conditional direction misses | lines 56–63

if (t->bi.br_flags & BR_CONDITIONAL) {
    ++conditional;

            // count a direction misprediction

            dmiss += u->direction_prediction () != t->taken;

        }

predict.cc | main: exact counts and MPKI output | lines 77–80

if (argc == 3) {
    printf("Conditional branches: %lld\nDirection misses: %lld\n", conditional, dmiss);
}
printf ("%0.3f MPKI\n", 1000.0 * (dmiss / 1e8));

# 3 Results

## 3.1 Experimental Setup

Each of the 20 supplied traces represents 100,000,000 executed instructions. Each run starts with a new predictor in the initial state listed in Section 2. All branches are passed through predict() and update(); only conditional direction errors contribute to MPKI. Lower MPKI is better.

MPKI = 1,000 × direction misses / 100,000,000 = direction misses / 100,000.

The denominator is the executed instruction count, not the number of conditional branches. The following table gives the exact branch and miss counts; MPKI is rounded to three decimal places.

## 3.2 Benchmark Results

| Benchmark | Conditional branches | Direction misses | MPKI |
| --- | --- | --- | --- |
| 164.gzip | 15,114,596 | 1,294,987 | 12.950 |
| 175.vpr | 12,827,221 | 1,682,106 | 16.821 |
| 176.gcc | 17,083,242 | 1,979,917 | 19.799 |
| 181.mcf | 21,700,420 | 2,007,479 | 20.075 |
| 186.crafty | 8,927,032 | 1,081,318 | 10.813 |
| 197.parser | 14,190,746 | 1,261,126 | 12.611 |
| 201.compress | 11,764,885 | 860,639 | 8.606 |
| 202.jess | 14,154,055 | 256,142 | 2.561 |
| 205.raytrace | 12,201,864 | 408,923 | 4.089 |
| 209.db | 13,191,770 | 527,170 | 5.272 |
| 213.javac | 12,986,593 | 328,799 | 3.288 |
| 222.mpegaudio | 13,021,205 | 325,094 | 3.251 |
| 227.mtrt | 12,512,685 | 390,366 | 3.904 |
| 228.jack | 11,926,271 | 568,496 | 5.685 |
| 252.eon | 7,724,960 | 383,095 | 3.831 |
| 253.perlbmk | 13,537,118 | 737,917 | 7.379 |
| 254.gap | 13,838,041 | 575,612 | 5.756 |
| 255.vortex | 11,073,761 | 222,843 | 2.228 |
| 256.bzip2 | 24,586,276 | 14,403 | 0.144 |
| 300.twolf | 13,098,893 | 2,423,827 | 24.238 |
| All traces / mean | 275,461,634 | 17,330,259 | 8.665 |

The final row sums the conditional branches and direction misses and reports mean MPKI. Because every trace has the same instruction count, the arithmetic mean of the per-trace MPKI values also equals aggregate MPKI over 2,000,000,000 instructions.

Mean MPKI = 17,330,259 / 2,000,000 = 8.6651295 ≈ 8.665.

The mean is calculated from exact miss counts before rounding the displayed per-trace values.

## 3.3 Analysis

The hybrid predictor averages 8.665 direction mispredictions per thousand instructions. Performance varies across the benchmarks: 256.bzip2 has the lowest MPKI (0.144), followed by 255.vortex (2.228) and 202.jess (2.561). The highest values occur in 300.twolf (24.238), 181.mcf (20.075), and 176.gcc (19.799).

Across all traces, 17,330,259 of 275,461,634 conditional directions are mispredicted, giving an aggregate conditional prediction accuracy of 93.709%. This accuracy uses conditional branches as its denominator, whereas MPKI uses all executed instructions.

The variation shows that the same predictor configuration handles the supplied branch streams with different effectiveness. Repeated address/history combinations can benefit from a trained global entry, while global misses rely on the bimodal table. Finite counters, limited history, and table aliasing can contribute to errors; the aggregate counts alone do not identify which factor dominates a particular benchmark.

# 4 What Works and Limitations

## 4.1 Implemented Behavior

The implementation provides bimodal prediction, tagged two-way global lookup, global-hit selection, saturating counter training, invalid-first allocation, LRU replacement, and conditional history updates. All supplied traces complete and produce the direction statistics shown in Section 3.

## 4.2 Model Scope and Limitations

This is the assignment’s simplified Pentium M–inspired direction predictor. Loop prediction, branch target prediction, the path information register, and its specialized hardware hash are omitted as permitted by the assignment. The two-way global organization is also explicitly permitted.

The predictor has finite tables and a 15-bit outcome history. Different branches or histories can share an index or tag, so aliasing and replacement may lose useful state. Every run includes cold-start training. The trace interface resolves each branch before the next prediction; speculative history, pipeline timing, and the power or cycle cost of a misprediction are not modeled.

# 5 References

[1] Assignment 1: Branch Predictor Implementation, programming-branch-predictor.pdf. Architecture diagram, required scope, and Appendix B worked hybrid-predictor examples.

[2] CBP2 Infrastructure v2, doc/index.html and supplied C++ source. Predictor interface, trace semantics, and MPKI definition.

The predictor tables and replacement logic are implemented directly with C++ arrays. No external cache implementation is used.