# Tomasulo Algorithm Implementation

Programming Assignment: In-order issue width 1-4

# 1 Introduction

The supplied Python Tomasulo simulator was extended to support issuing up to one, two, three, or four instructions per cycle in program order. Instructions may execute out of order when their operands and functional units are available. Results are committed in program order.

The processor has five stages: ISSUE, EX, MEM, WB, and COMMIT. It uses a register alias table (RAT), a reorder buffer (ROB), reservation stations, a load/store queue, a branch predictor, and one common data bus (CDB). ISSUE includes instruction fetch and decode.

# 2 Code Changes

## 2.1 Input configuration and issue-width validation

The original reader loaded every line as an instruction. The modified parser separates hardware settings, initial values, and instructions. Issue width is specified as Issue width = N, accepts integers from 1 to 4, and defaults to 1. Invalid widths are rejected.

The input also configures ROB capacity, reservation-station and load/store-queue sizes, functional-unit counts, execution latencies, and memory latency. Instructions and configuration names are case insensitive. Registers and memory words start at zero unless initialized in the input. Supported instructions are Ld, Sd, Beq, Bne, Add, Sub, Addi, Add.d, Sub.d, and Mult.d.

Floating-point registers and memory words use IEEE 754 binary32 (32-bit single precision). Initial FP values are converted by float32, which packs and unpacks the IEEE single-precision representation using the standard-library struct module. Rounding is to nearest, with ties to even. Initial values outside the finite binary32 range are rejected.

### 2.1.1 Initial Version

Initial Version | supplied init.py | read_instruction | lines 58-61

def read_instruction(codefile):
    with open(codefile) as f:
            instructions = f.read().splitlines()
    return instructions

### 2.1.2 Modified Version

Modified Version | code/init.py | Config default (class excerpt) | lines 13-14

class Config:
    issue_width: int = 1

Modified Version | code/init.py | Config.validate | lines 30-35

def validate(self):
    for name, value in vars(self).items():
        if type(value) is not int or value < 1:
            raise ValueError(f"{name} must be a positive integer")
    if self.issue_width > 4:
        raise ValueError("issue width must be between 1 and 4")

Modified Version | code/init.py | parse_program: issue-width / ROB setting | lines 195-205

setting = re.match(r"^(issue\s+width|rob\s+entries)\b", line, re.I)
if setting:
    key = " ".join(setting[1].lower().split())
    match = re.fullmatch(r"(?:issue\s+width|rob\s+entries)\s*=\s*([+-]?\d+)", line, re.I)
    if not match:
        raise ValueError(f"{key} requires an integer after '='")
    if key in seen:
        raise ValueError(f"duplicate {key} setting")
    seen.add(key)
    setattr(config, "issue_width" if key == "issue width" else "rob_entries", int(match[1]))
    continue

Modified Version | code/init.py | float32: single-precision conversion | lines 126-131

def float32(value):
    """Round to IEEE 754 binary32, using round-to-nearest, ties-to-even."""
    try:
        return struct.unpack("!f", struct.pack("!f", value))[0]
    except OverflowError:
        return math.copysign(math.inf, value)

Modified Version | code/init.py | parse_program: FP register and memory initialization | lines 231-234

if not key.startswith("R"):
    value = float32(value)
    if not math.isfinite(value):
        raise ValueError("initial values must be finite in binary32")

## 2.2 Multiple issue in program order

The original issue function fetched one instruction per call. The modified stage issues up to the configured width in program order and stops at the first unavailable ROB, reservation-station, or load/store-queue entry. Each instruction reads its source operands before renaming its destination. The updated RAT is used by the next instruction, preserving dependencies within the same issue cycle.

### 2.2.1 Initial Version

Initial Version | supplied issue.py | issue: single instruction fetch | lines 171-179

def issue(cycle, PC, instructions, ROB, size_ROB, 
        rs_int_adder, 
        rs_fp_adder, 
        rs_fp_multi, 
        ld_sd_queue, size_ld_sd_queue, 
        rat_int, rat_fp):
    # fetch 1 instruction 
    ins = instructions[PC.PC]
    op = ins.split(' ')[0]

Initial Version | supplied issue.py | issue: integer resource checks and allocation | lines 196-207

elif (op=='Add')|(op=='Addi')|(op=='Sub'):
    # check space
    if (len(ROB)<size_ROB)&(check_rs_space(rs_int_adder)>=0):
        # put ins into ROB
        put_ins_into_ROB(ROB, size_ROB, PC, cycle, ins, '')
        # put ins into rs_int_adder
        index = check_rs_space(rs_int_adder)
        put_ins_into_rs(rs_int_adder, index, ins, ROB, rat_int, rat_fp)
        # update RAT
        update_rat(ROB, rat_int, rat_fp)
        # PC
        PC.PC +=1

### 2.2.2 Modified Version

Modified Version | code/issue.py | issue: width loop, checks, allocation and renaming | lines 15-38

def issue(sim):
    if sim.cycle < sim.fetch_resume:
        return
    for _ in range(sim.config.issue_width):
        if sim.pc == len(sim.program.instructions):
            break
        instruction = sim.program.instructions[sim.pc]
        group = instruction.group
        queue = sim.lsq if group == "address" else sim.rs[group]
        if len(sim.rob) >= sim.config.rob_entries or len(queue) >= sim.capacity[group]:
            sim.stats["issue_stall_cycles"] += 1
            break
        # Read sources BEFORE renaming this destination (e.g., Addi R1 R1 -1).
        operands = [source_operand(sim, reg) for reg in instruction.sources]
        tag = sim.next_tag
        sim.next_tag += 1
        entry = Entry(tag, instruction, operands, sim.cycle)
        sim.entries[tag] = entry
        sim.history.append(entry)
        sim.rob.append(tag)
        queue.append(tag)
        if instruction.dest and instruction.dest != "R0":
            sim.rat[instruction.dest] = tag
        sim.pc += 1

## 2.3 Register dependencies, write-back, and register commit

The original write-back replaced the RAT value directly. The modified code wakes waiting operands by producer tag and removes a RAT mapping only when the matching producer commits. This prevents an older result from replacing a younger register mapping. The register files contain 32 integer registers and 32 floating-point registers; R0 is permanently zero.

The single CDB broadcasts at most one result per cycle. Architectural registers are updated only at the ROB head, with at most one instruction retired per cycle. Increasing issue width changes issue capacity; CDB and commit bandwidth stay fixed.

### 2.3.1 Initial Version

Initial Version | supplied wb.py | boroadcast: result and RAT update | lines 19-31

def boroadcast(cdb, rat_int, rat_fp, 
                rs_int_adder, rs_fp_adder, rs_fp_multi,
                ld_sd_queue, ROB, cycle):
    # dest_tag
    dest_tag = cdb.dest_tag
    index = find_ROB_entry(ROB, dest_tag)
    reg_tag = ROB[index].dest_tag
    # rat_int
    if reg_tag[0] == 'R':
        rat_int[int(reg_tag[1:])] = cdb.value
    # rat_fp
    if reg_tag[0] == 'F':
        rat_fp[int(reg_tag[1:])] = cdb.value

Initial Version | supplied commit.py | modify_arch_reg | lines 18-24

def modify_arch_reg(entry, reg_int, reg_fp):
    if entry.dest_tag[0] == 'F':
        reg_fp[int(entry.dest_tag[1:])] = entry.value
    elif entry.dest_tag[0] == 'R':
        reg_int[int(entry.dest_tag[1:])] = entry.value
    else:
        pass

### 2.3.2 Modified Version

Modified Version | code/wb.py | write_back: result publication and operand wake-up | lines 10-23

_, tag, value = result
entry = sim.entries[tag]
entry.value = value
entry.ready = True
entry.ready_cycle = sim.cycle
entry.wb = sim.cycle
# Keep RAT producer tags until commit: an older completion must never
# overwrite a mapping installed by a younger writer to the same register.
for waiting in sim.entries.values():
    for operand in waiting.operands:
        if operand.tag == tag:
            operand.tag = None
            operand.value = value
            operand.available = sim.cycle

Modified Version | code/commit.py | retire: architectural registers and RAT | lines 5-12

def retire(sim, entry):
    assert sim.rob[0] == entry.tag
    sim.rob.popleft()
    dest = entry.instruction.dest
    if dest and dest != "R0":
        sim.registers[dest] = entry.value
        if sim.rat.get(dest) == entry.tag:
            del sim.rat[dest]

## 2.4 Functional-unit execution and cycle timing

The initial execution stage could advance a functional-unit counter twice in one cycle when dispatching a waiting operation. The modified scheduler selects ready instructions in age order within each functional-unit class and records completion as start cycle + latency - 1. Integer and address units are unpipelined. Each pipelined FP adder or multiplier can accept one new operation per cycle.

An instruction starts EX no earlier than the cycle after issue. A completed result becomes eligible for write-back in the following cycle, and a dependent instruction starts EX no earlier than the cycle after that write-back. These boundaries preserve the configured latencies and dependencies.

Add.d, Sub.d, and Mult.d results are rounded to binary32 before they enter the result buffer. The CDB, dependent instructions, and architectural registers therefore receive the same single-precision value. For example, 16777216 + 1 rounds to 16777216; subtracting 16777216 in the next instruction produces 0. Integer operations keep integer values.

### 2.4.1 Initial Version

Initial Version | supplied exe.py | fu_exe: advance execution cycles | lines 50-61

def fu_exe(fu, fu_results, ROB, time_fu, cycle, PC):
    if len(fu)!=0:
        for element in fu:
            element.cycle += 1
        # write starting cycle 
        if fu[-1].cycle == 1:
            index = find_ROB_entry(ROB, fu[-1].dest_tag)
            ROB[index].exe.append(cycle)
        # finish cycle
        if fu[0].cycle == time_fu:
            index = find_ROB_entry(ROB, fu[0].dest_tag)
            ROB[index].exe.append(cycle)

Initial Version | supplied exe.py | exe: existing functional-unit advancement | lines 120-123

# functional units 
fu_exe(fu_int_adder, results_buffer, ROB, time_fu_int_adder, cycle, PC)
fu_exe(fu_fp_adder, results_buffer, ROB, time_fu_fp_adder, cycle, PC)
fu_exe(fu_fp_multi, results_buffer, ROB, time_fu_fp_multi, cycle, PC)

Initial Version | supplied exe.py | exe: integer dispatch and second fu_exe call | lines 140-148

if check_valid_ins_in_rs(rs_int_adder)>=0:
    index = check_valid_ins_in_rs(rs_int_adder)
    fu_int_adder.append(fu_entry())
    add_entry_into_fu(fu_int_adder, rs_int_adder[index])
    # check if it's a waiting instruction
    if ROB[find_ROB_entry(ROB, rs_int_adder[index].dest_tag)].issue[0] < cycle:
        fu_exe(fu_int_adder, results_buffer, ROB, time_fu_int_adder, cycle, PC)
    # remove ins from rs 
    rs_int_adder[index].busy = 0

### 2.4.2 Modified Version

Modified Version | code/exe.py | execute: ready selection, FU capacity and completion | lines 5-32

def execute(sim):
    for group in ("int", "add", "mul", "address"):
        queue = sim.lsq if group == "address" else sim.rs[group]
        active = sum(job.group == group for job in sim.executing)
        slots = sim.units[group]
        if group in ("int", "address"):
            slots -= active
        for tag in list(queue):
            if slots <= 0:
                break
            entry = sim.entries[tag]
            if entry.issue >= sim.cycle or entry.ex_start is not None:
                continue
            operands = entry.operands[:1] if group == "address" else entry.operands
            if not all(operand.ready(sim.cycle) for operand in operands):
                continue
            entry.ex_start = sim.cycle
            sim.executing.append(Execution(tag, group, sim.cycle + sim.latency[group] - 1))
            if group != "address":
                queue.remove(tag)
            slots -= 1
    # Complete in age order so an older redirect wins over younger work.
    for job in sorted(list(sim.executing), key=lambda item: item.tag):
        if job.end != sim.cycle or job.tag not in sim.entries:
            continue
        sim.executing.remove(job)
        entry = sim.entries[job.tag]
        entry.ex_end = sim.cycle

Modified Version | code/exe.py | execute: round FP arithmetic before result publication | lines 61-71

if op == "addi":
    value = values[0] + instruction.immediate
elif op in ("add", "add.d"):
    value = values[0] + values[1]
elif op in ("sub", "sub.d"):
    value = values[0] - values[1]
else:
    value = values[0] * values[1]
if job.group in ("add", "mul"):
    value = float32(value)
sim.results.append((sim.cycle, entry.tag, value))

## 2.5 Load/store forwarding and in-order store commit

The initial forwarding search stopped at the nearest store even when its address did not match, and its store-dispatch condition tested a function object instead of calling it. The modified memory stage selects the youngest older store with a matching address and forwards its value in one cycle when the store data is ready. A matching store with unavailable data makes the load wait.

Loads and stores enter the load/store queue in program order. A dedicated address unit calculates their effective byte addresses in EX without using the integer ALU. Data memory contains 256 bytes represented as 64 words and has one nonpipelined port. Loads leave the queue after receiving data; stores write memory and leave the queue only when committing at the ROB head. Stores write binary32 values, and both memory reads and store-to-load forwarding return binary32 values.

A load without a matching resolved store may access memory even when older store addresses are unresolved. If an older store later reveals that the load read an incorrect value, the simulator squashes that load and younger instructions and re-executes them to preserve memory ordering.

### 2.5.1 Initial Version

Initial Version | supplied mem.py | forward_check_from_sd | lines 19-27

def forward_check_from_sd(ld_sd_queue, index):
    data = []
    for i in reversed(range(index)):
        if (ld_sd_queue[i].op=='Sd'):
            if (ld_sd_queue[i].address == ld_sd_queue[index].address):
                if (type(ld_sd_queue[i].data)==int)|(type(ld_sd_queue[i].data)==float):
                    data.append(ld_sd_queue[i].data)
            break
    return data

Initial Version | supplied mem.py | mem: store dispatch | lines 127-136

if (ld_sd_queue[0].op=='Sd'):
    # check if this Sd is ready and to be commited 
    if (ld_sd_queue[0].ready==1):
        # check to be committed in next cycle
        if (check_if_sd_committable):
            # already have data in Sd
            if (type(ld_sd_queue[0].data)==int)|(type(ld_sd_queue[0].data)==float):
                # put the Sd entry into ld_sd_mem
                entry = ld_sd_queue.popleft()
                put_entry_into_ld_sd_mem(ld_sd_mem, entry)

Initial Version | supplied commit.py | commit: handling the store entry | lines 39-47

elif len(ROB[0].commit)!=0: # Sd
    entry = ROB.popleft()
    print_ROB(entry, instructions)
    if len(ROB)>0:
        if len(ROB[0].cdb)!=0: # broadcasted instructions
            ROB[0].commit.append(cycle+1)
            entry = ROB.popleft()
            modify_arch_reg(entry, reg_int, reg_fp)
            print_ROB(entry, instructions)

### 2.5.2 Modified Version

Modified Version | code/mem.py | memory: match older stores and forward data | lines 14-26

older_matches = [sim.entries[older] for older in sim.lsq
                 if older < tag and sim.entries[older].instruction.op == "sd"
                 and sim.entries[older].address == entry.address]
source = max(older_matches, key=lambda candidate: candidate.tag, default=None)
if source:
    if (forwarding_used or source.address_cycle >= sim.cycle
            or not source.operands[1].ready(sim.cycle)):
        continue
    entry.mem_start = sim.cycle
    entry.forwarded_from = source.tag
    sim.forwarding.append(MemoryAccess(tag, sim.cycle, forwarded=True,
                                       value=source.operands[1].value))
    forwarding_used = True

Modified Version | code/commit.py | commit: ROB head and store memory transaction | lines 21-40

def commit(sim):
    if not sim.rob:
        return
    entry = sim.entries[sim.rob[0]]
    if entry.fault:
        raise ValueError(f"PC {entry.instruction.pc * 4}: {entry.fault}")
    if entry.instruction.op == "sd":
        if entry.commit_start is not None:
            return
        if (entry.address_cycle is None or entry.address_cycle >= sim.cycle
                or not entry.operands[1].ready(sim.cycle)):
            return
        if sim.memory_access is not None or sim.cycle < sim.memory_available:
            return
        entry.commit_start = sim.cycle
        entry.mem_start = sim.cycle
        sim.memory_access = MemoryAccess(entry.tag, sim.cycle + sim.config.memory_cycles - 1,
                                         store=True, value=entry.operands[1].value)
    elif entry.ready and entry.ready_cycle < sim.cycle:
        retire(sim, entry)

Modified Version | code/mem.py | memory: complete store and retire | lines 43-45

if access.store:
    sim.memory[entry.address] = float32(access.value)
    retire(sim, entry)

Modified Version | code/mem.py | memory: round loaded and forwarded data | lines 46-49

else:
    value = access.value if access.forwarded else sim.memory[entry.address]
    sim.results.append((sim.cycle, entry.tag, float32(value)))
    sim.lsq.remove(entry.tag)

## 2.6 Branch prediction and recovery

The initial branch handling stopped fetch at Bne until EX determined the next PC. The modified implementation predicts in ISSUE using an eight-entry branch target buffer (BTB) with a one-bit direction predictor. The index is (byte_PC >> 2) & 7, the least significant three bits of the word address. A BTB miss predicts not taken. Beq and Bne use the integer reservation stations and resolve at the end of EX.

Resolution updates the prediction and target. On a misprediction, younger ROB, reservation-station, and load/store-queue entries are removed, including their operand tags. Younger active operations and pending results are discarded, and the RAT is rebuilt from surviving instructions. If the misprediction is detected in cycle n, correct fetching resumes in cycle n+2. Branch targets use PC + 4 + (offset << 2), with offsets measured in words.

### 2.6.1 Initial Version

Initial Version | supplied issue.py | issue: Bne stops fetching | lines 235-244

elif (op=='Bne'):
    # check space
    if (len(ROB)<size_ROB)&(check_rs_space(rs_int_adder)>=0):
        # put ins into ROB
        put_ins_into_ROB(ROB, size_ROB, PC, cycle, ins, '')
        # put ins into rs_int_adder
        index = check_rs_space(rs_int_adder)
        put_bne_into_rs(rs_int_adder, index, ins, rat_int, rat_fp, ROB)
        # PC
        PC.valid = 0

Initial Version | supplied exe.py | fu_exe: Bne resolution and PC update | lines 71-80

elif (fu[0].op=='Bne'):
    fu_results.pop()
    if fu[0].value1 == fu[0].value2:
        PC.PC += 1
        PC.valid = 1
    else:
        index = find_ROB_entry(ROB, fu[0].dest_tag)
        offset = ROB[index].dest_tag
        PC.PC = int(PC.PC + 1 + offset/4)
        PC.valid = 1

### 2.6.2 Modified Version

Modified Version | code/issue.py | issue: prediction | lines 39-44

if instruction.op in ("beq", "bne"):
    predictor = sim.btb[instruction.pc & 7]
    if predictor and predictor.pc == instruction.pc and predictor.taken:
        entry.predicted_taken = True
        sim.pc = predictor.target
    entry.predicted_next = sim.pc

Modified Version | code/exe.py | execute: resolution and recovery request | lines 49-59

elif op in ("beq", "bne"):
    taken = (values[0] == values[1]) if op == "beq" else (values[0] != values[1])
    target = instruction.pc + 1 + instruction.immediate
    actual_next = target if taken else instruction.pc + 1
    sim.btb[instruction.pc & 7] = Prediction(instruction.pc, target, taken)
    entry.ready = True
    entry.ready_cycle = sim.cycle
    sim.stats["branches"] += 1
    if entry.predicted_taken != taken or entry.predicted_next != actual_next:
        sim.stats["mispredictions"] += 1
        sim.squash_from(entry.tag + 1, actual_next, "branch misprediction")

Modified Version | code/simulator.py | squash_from: discard younger work (new function) | lines 47-64

def squash_from(self, first_tag, pc, reason):
    removed = {tag for tag in self.rob if tag >= first_tag}
    for tag in removed:
        entry = self.entries.pop(tag)
        entry.status = "squashed"
        entry.squash_cycle = self.cycle
        entry.squash_reason = reason
    self.rob = deque(tag for tag in self.rob if tag not in removed)
    for group in self.rs:
        self.rs[group] = [tag for tag in self.rs[group] if tag not in removed]
    self.lsq = [tag for tag in self.lsq if tag not in removed]
    self.executing = [job for job in self.executing if job.tag not in removed]
    self.results = [result for result in self.results if result[1] not in removed]
    self.forwarding = [access for access in self.forwarding if access.tag not in removed]
    if self.memory_access and self.memory_access.tag in removed:
        assert not self.memory_access.store
        self.memory_access = None
        self.memory_available = self.cycle + 1

Modified Version | code/simulator.py | squash_from: restore mappings and n+2 restart | lines 65-72

# Rebuild mappings from surviving instructions in program order.
self.rat.clear()
for tag in self.rob:
    dest = self.entries[tag].instruction.dest
    if dest and dest != "R0":
        self.rat[dest] = tag
self.pc = pc
self.fetch_resume = self.cycle + 2

## 2.7 Instruction status and final register/memory output

The initial output printed a timing row when an instruction committed. The modified simulator prints each dynamic instruction with its ISSUE, EX, MEM, WB, and COMMIT cycles and its committed or squashed status. It also prints all 32 integer and 32 floating-point register values and every nonzero memory word with its byte address. Floating-point output uses up to nine significant decimal digits so distinct binary32 values remain distinguishable.

### 2.7.1 Initial Version

Initial Version | supplied commit.py | print_ROB | lines 9-16

def print_ROB(entry, instructions):
    item = instructions[entry.PC].ljust(30)
    item += str(entry.issue).ljust(15) 
    item += str(entry.exe).ljust(15) 
    item += str(entry.mem).ljust(15)
    item += str(entry.cdb).ljust(15)
    item += str(entry.commit)
    print(item)

### 2.7.2 Modified Version

Modified Version | code/print_status.py | format_result: final registers and memory | lines 31-39

lines.extend(["", "REGISTER VALUES"])
for kind in ("R", "F"):
    for start in range(0, 32, 8):
        lines.append("  ".join(f"{kind}{i}={number(result['registers'][f'{kind}{i}'])}"
                               for i in range(start, start + 8)))
lines.extend(["", "NON-ZERO MEMORY VALUES", "Address   Value"])
lines.extend(f"{address:<9} {number(value)}" for address, value in result["memory"].items())
if not result["memory"]:
    lines.append("(none)")

Modified Version | code/print_status.py | number: preserve binary32 precision in output | lines 12-14

def number(value):
    """Nine significant digits distinguish binary32 values in text output."""
    return format(value, ".9g") if isinstance(value, float) else str(value)

# 3 Results

Two benchmarks were executed at issue widths 1, 2, 3, and 4: an independent instruction sequence and an array loop with dependencies, memory accesses, and branches. Within each benchmark, only issue width changes; hardware resources, latencies, instructions, and initial values are fixed. Final register and memory values are compared with the expected calculations.

Cycle ranges include both endpoints. A dash indicates a stage that does not apply. Tables list committed instructions; repeated instructions represent loop iterations. For stores, the COMMIT range covers the memory write. The single CDB and commit bandwidth remain unchanged when issue width increases.

## 3.1 Test Case 1: Independent instructions

This benchmark checks simultaneous issue and execution across multiple functional units. With R1=2, R2=3, F1=2, and F2=3, the expected results are R10=2+3=5, R11=3-2=1, R12=9, F10=F11=2*3=6, F12=F13=2+3=5, and F14=3-2=1.

### 3.1.1 Configuration and instructions

# Unit              RS  EX  MEM  FUs
Integer adder       8   2       2
FP adder            8   3       2
FP multiplier       8   8       2
Load/store unit     8   1   3   1
ROB entries = 32
Issue width = 1
R1=2, R2=3, F1=2, F2=3
Mult.d F10 F1 F2
Mult.d F11 F1 F2
Add.d F12 F1 F2
Add R10 R1 R2
Add.d F13 F1 F2
Sub R11 R2 R1
Addi R12 R0 9
Sub.d F14 F2 F1

### 3.1.2 Results for each issue width

| Issue width | Total cycles | Final register and memory values |
| --- | --- | --- |
| 1 | 20 | Correct |
| 2 | 19 | Correct |
| 3 | 19 | Correct |
| 4 | 18 | Correct |

Increasing issue width reduces completion time from 20 cycles at width 1 to 18 cycles at width 4. At width 4, the first four instructions all issue in cycle 1. Execution latency, the single CDB, and single-instruction commit limit the improvement. All four widths produce the same final register values and leave memory unchanged.

### 3.1.3 Final register and memory values

R0=0  R1=2  R2=3  R3=0  R4=0  R5=0  R6=0  R7=0
R8=0  R9=0  R10=5  R11=1  R12=9  R13=0  R14=0  R15=0
R16=0  R17=0  R18=0  R19=0  R20=0  R21=0  R22=0  R23=0
R24=0  R25=0  R26=0  R27=0  R28=0  R29=0  R30=0  R31=0
F0=0  F1=2  F2=3  F3=0  F4=0  F5=0  F6=0  F7=0
F8=0  F9=0  F10=6  F11=6  F12=5  F13=5  F14=1  F15=0
F16=0  F17=0  F18=0  F19=0  F20=0  F21=0  F22=0  F23=0
F24=0  F25=0  F26=0  F27=0  F28=0  F29=0  F30=0  F31=0

Nonzero memory values: none.

### 3.1.4 Instruction timing - issue width 1

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Mult.d F10 F1 F2 | 1 | 2-9 | - | 11 | 12 |
| Mult.d F11 F1 F2 | 2 | 3-10 | - | 13 | 14 |
| Add.d F12 F1 F2 | 3 | 4-6 | - | 7 | 15 |
| Add R10 R1 R2 | 4 | 5-6 | - | 8 | 16 |
| Add.d F13 F1 F2 | 5 | 6-8 | - | 9 | 17 |
| Sub R11 R2 R1 | 6 | 7-8 | - | 10 | 18 |
| Addi R12 R0 9 | 7 | 8-9 | - | 12 | 19 |
| Sub.d F14 F2 F1 | 8 | 9-11 | - | 14 | 20 |

### 3.1.5 Instruction timing - issue width 2

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Mult.d F10 F1 F2 | 1 | 2-9 | - | 11 | 12 |
| Mult.d F11 F1 F2 | 1 | 2-9 | - | 12 | 13 |
| Add.d F12 F1 F2 | 2 | 3-5 | - | 6 | 14 |
| Add R10 R1 R2 | 2 | 3-4 | - | 5 | 15 |
| Add.d F13 F1 F2 | 3 | 4-6 | - | 8 | 16 |
| Sub R11 R2 R1 | 3 | 4-5 | - | 7 | 17 |
| Addi R12 R0 9 | 4 | 5-6 | - | 9 | 18 |
| Sub.d F14 F2 F1 | 4 | 5-7 | - | 10 | 19 |

### 3.1.6 Instruction timing - issue width 3

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Mult.d F10 F1 F2 | 1 | 2-9 | - | 11 | 12 |
| Mult.d F11 F1 F2 | 1 | 2-9 | - | 12 | 13 |
| Add.d F12 F1 F2 | 1 | 2-4 | - | 5 | 14 |
| Add R10 R1 R2 | 2 | 3-4 | - | 6 | 15 |
| Add.d F13 F1 F2 | 2 | 3-5 | - | 8 | 16 |
| Sub R11 R2 R1 | 2 | 3-4 | - | 7 | 17 |
| Addi R12 R0 9 | 3 | 5-6 | - | 9 | 18 |
| Sub.d F14 F2 F1 | 3 | 4-6 | - | 10 | 19 |

### 3.1.7 Instruction timing - issue width 4

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Mult.d F10 F1 F2 | 1 | 2-9 | - | 10 | 11 |
| Mult.d F11 F1 F2 | 1 | 2-9 | - | 11 | 12 |
| Add.d F12 F1 F2 | 1 | 2-4 | - | 5 | 13 |
| Add R10 R1 R2 | 1 | 2-3 | - | 4 | 14 |
| Add.d F13 F1 F2 | 2 | 3-5 | - | 7 | 15 |
| Sub R11 R2 R1 | 2 | 3-4 | - | 6 | 16 |
| Addi R12 R0 9 | 2 | 4-5 | - | 8 | 17 |
| Sub.d F14 F2 F1 | 2 | 3-5 | - | 9 | 18 |

## 3.2 Test Case 2: Array loop with dependencies and memory

This test uses the supplied array computation. Each of three iterations updates a destination word using Mem[R2] = Mem[R2] + 3 * Mem[R1]. The expected values are Mem[32]=4+3*1=7, Mem[28]=5+3*2=11, and Mem[24]=6+3*3=15. It exercises loads, stores, dependent arithmetic, register reuse, and loop branches.

The branch displacement is -8 words, following target = PC + 4 + (offset << 2). This returns from the eighth instruction to the first. After three iterations, R1=0 and R2=20. The last iteration leaves F2=3, F4=9, and F6=15; the instruction after the loop sets F20=F2+F2=6.

### 3.2.1 Configuration and instructions

# Unit              RS  EX  MEM  FUs
Integer adder 4 1 1
FP adder 3 4 1
FP multiplier 2 15 1
Load/store unit 5 1 5 1
ROB entries = 64
Issue width = 1
R1=12, R2=32, F20=3.0
Mem[4]=3.0, Mem[8]=2.0, Mem[12]=1.0
Mem[24]=6.0, Mem[28]=5.0, Mem[32]=4.0
Ld F2 0(R1)
Mult.d F4 F2 F20
Ld F6 0(R2)
Add.d F6 F4 F6
Sd F6 0(R2)
Addi R1 R1 -4
Addi R2 R2 -4
Bne R1 R0 -8
Add.d F20 F2 F2

### 3.2.2 Results for each issue width

| Issue width | Total cycles | Final register and memory values |
| --- | --- | --- |
| 1 | 66 | Correct |
| 2 | 65 | Correct |
| 3 | 65 | Correct |
| 4 | 65 | Correct |

Completion time decreases from 66 cycles at width 1 to 65 cycles at widths 2-4. The final values are identical at every width. Data dependencies, long multiplication latency, and the single memory port limit the benefit of wider issue.

### 3.2.3 Final register and memory values

R0=0  R1=0  R2=20  R3=0  R4=0  R5=0  R6=0  R7=0
R8=0  R9=0  R10=0  R11=0  R12=0  R13=0  R14=0  R15=0
R16=0  R17=0  R18=0  R19=0  R20=0  R21=0  R22=0  R23=0
R24=0  R25=0  R26=0  R27=0  R28=0  R29=0  R30=0  R31=0
F0=0  F1=0  F2=3  F3=0  F4=9  F5=0  F6=15  F7=0
F8=0  F9=0  F10=0  F11=0  F12=0  F13=0  F14=0  F15=0
F16=0  F17=0  F18=0  F19=0  F20=6  F21=0  F22=0  F23=0
F24=0  F25=0  F26=0  F27=0  F28=0  F29=0  F30=0  F31=0

| Nonzero memory address | Final value |
| --- | --- |
| 4 | 3 |
| 8 | 2 |
| 12 | 1 |
| 24 | 15 |
| 28 | 11 |
| 32 | 7 |

### 3.2.4 Instruction timing - issue width 1

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Ld F2 0(R1) | 1 | 2 | 3-7 | 8 | 9 |
| Mult.d F4 F2 F20 | 2 | 9-23 | - | 24 | 25 |
| Ld F6 0(R2) | 3 | 4 | 8-12 | 13 | 26 |
| Add.d F6 F4 F6 | 4 | 25-28 | - | 29 | 30 |
| Sd F6 0(R2) | 5 | 6 | 34-38 | - | 34-38 |
| Addi R1 R1 -4 | 6 | 7 | - | 9 | 39 |
| Addi R2 R2 -4 | 7 | 8 | - | 10 | 40 |
| Bne R1 R0 -8 | 8 | 10 | - | - | 41 |
| Ld F2 0(R1) | 12 | 13 | 14-18 | 19 | 42 |
| Mult.d F4 F2 F20 | 13 | 20-34 | - | 35 | 43 |
| Ld F6 0(R2) | 14 | 15 | 19-23 | 25 | 44 |
| Add.d F6 F4 F6 | 15 | 36-39 | - | 40 | 45 |
| Sd F6 0(R2) | 16 | 17 | 46-50 | - | 46-50 |
| Addi R1 R1 -4 | 17 | 18 | - | 20 | 51 |
| Addi R2 R2 -4 | 18 | 19 | - | 21 | 52 |
| Bne R1 R0 -8 | 19 | 21 | - | - | 53 |
| Ld F2 0(R1) | 20 | 21 | 24-28 | 30 | 54 |
| Mult.d F4 F2 F20 | 21 | 31-45 | - | 46 | 55 |
| Ld F6 0(R2) | 22 | 23 | 29-33 | 34 | 56 |
| Add.d F6 F4 F6 | 23 | 47-50 | - | 51 | 57 |
| Sd F6 0(R2) | 24 | 25 | 58-62 | - | 58-62 |
| Addi R1 R1 -4 | 25 | 26 | - | 27 | 63 |
| Addi R2 R2 -4 | 26 | 27 | - | 28 | 64 |
| Bne R1 R0 -8 | 27 | 28 | - | - | 65 |
| Add.d F20 F2 F2 | 30 | 31-34 | - | 36 | 66 |

### 3.2.5 Instruction timing - issue width 2

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Ld F2 0(R1) | 1 | 2 | 3-7 | 8 | 9 |
| Mult.d F4 F2 F20 | 1 | 9-23 | - | 24 | 25 |
| Ld F6 0(R2) | 2 | 3 | 8-12 | 13 | 26 |
| Add.d F6 F4 F6 | 2 | 25-28 | - | 29 | 30 |
| Sd F6 0(R2) | 3 | 4 | 33-37 | - | 33-37 |
| Addi R1 R1 -4 | 3 | 4 | - | 5 | 38 |
| Addi R2 R2 -4 | 4 | 5 | - | 6 | 39 |
| Bne R1 R0 -8 | 4 | 6 | - | - | 40 |
| Ld F2 0(R1) | 8 | 9 | 13-17 | 18 | 41 |
| Mult.d F4 F2 F20 | 8 | 19-33 | - | 35 | 42 |
| Ld F6 0(R2) | 9 | 10 | 18-22 | 23 | 43 |
| Add.d F6 F4 F6 | 9 | 36-39 | - | 40 | 44 |
| Sd F6 0(R2) | 10 | 11 | 45-49 | - | 45-49 |
| Addi R1 R1 -4 | 10 | 11 | - | 12 | 50 |
| Addi R2 R2 -4 | 11 | 12 | - | 14 | 51 |
| Bne R1 R0 -8 | 11 | 13 | - | - | 52 |
| Ld F2 0(R1) | 12 | 13 | 23-27 | 28 | 53 |
| Mult.d F4 F2 F20 | 12 | 29-43 | - | 44 | 54 |
| Ld F6 0(R2) | 17 | 18 | 28-32 | 33 | 55 |
| Add.d F6 F4 F6 | 17 | 45-48 | - | 49 | 56 |
| Sd F6 0(R2) | 22 | 23 | 57-61 | - | 57-61 |
| Addi R1 R1 -4 | 22 | 23 | - | 25 | 62 |
| Addi R2 R2 -4 | 23 | 24 | - | 26 | 63 |
| Bne R1 R0 -8 | 23 | 26 | - | - | 64 |
| Add.d F20 F2 F2 | 28 | 29-32 | - | 34 | 65 |

### 3.2.6 Instruction timing - issue width 3

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Ld F2 0(R1) | 1 | 2 | 3-7 | 8 | 9 |
| Mult.d F4 F2 F20 | 1 | 9-23 | - | 24 | 25 |
| Ld F6 0(R2) | 1 | 3 | 8-12 | 13 | 26 |
| Add.d F6 F4 F6 | 2 | 25-28 | - | 29 | 30 |
| Sd F6 0(R2) | 2 | 4 | 33-37 | - | 33-37 |
| Addi R1 R1 -4 | 2 | 3 | - | 4 | 38 |
| Addi R2 R2 -4 | 3 | 4 | - | 5 | 39 |
| Bne R1 R0 -8 | 3 | 5 | - | - | 40 |
| Ld F2 0(R1) | 7 | 8 | 13-17 | 18 | 41 |
| Mult.d F4 F2 F20 | 7 | 19-33 | - | 35 | 42 |
| Ld F6 0(R2) | 7 | 9 | 18-22 | 23 | 43 |
| Add.d F6 F4 F6 | 8 | 36-39 | - | 40 | 44 |
| Sd F6 0(R2) | 8 | 10 | 45-49 | - | 45-49 |
| Addi R1 R1 -4 | 8 | 9 | - | 10 | 50 |
| Addi R2 R2 -4 | 9 | 10 | - | 11 | 51 |
| Bne R1 R0 -8 | 9 | 11 | - | - | 52 |
| Ld F2 0(R1) | 12 | 13 | 23-27 | 28 | 53 |
| Mult.d F4 F2 F20 | 12 | 29-43 | - | 44 | 54 |
| Ld F6 0(R2) | 17 | 18 | 28-32 | 33 | 55 |
| Add.d F6 F4 F6 | 17 | 45-48 | - | 49 | 56 |
| Sd F6 0(R2) | 22 | 23 | 57-61 | - | 57-61 |
| Addi R1 R1 -4 | 22 | 23 | - | 25 | 62 |
| Addi R2 R2 -4 | 22 | 24 | - | 26 | 63 |
| Bne R1 R0 -8 | 23 | 26 | - | - | 64 |
| Add.d F20 F2 F2 | 28 | 29-32 | - | 34 | 65 |

### 3.2.7 Instruction timing - issue width 4

| Instruction | ISSUE | EX | MEM | WB | COMMIT |
| --- | --- | --- | --- | --- | --- |
| Ld F2 0(R1) | 1 | 2 | 3-7 | 8 | 9 |
| Mult.d F4 F2 F20 | 1 | 9-23 | - | 24 | 25 |
| Ld F6 0(R2) | 1 | 3 | 8-12 | 13 | 26 |
| Add.d F6 F4 F6 | 1 | 25-28 | - | 29 | 30 |
| Sd F6 0(R2) | 2 | 4 | 33-37 | - | 33-37 |
| Addi R1 R1 -4 | 2 | 3 | - | 4 | 38 |
| Addi R2 R2 -4 | 2 | 4 | - | 5 | 39 |
| Bne R1 R0 -8 | 2 | 5 | - | - | 40 |
| Ld F2 0(R1) | 7 | 8 | 13-17 | 18 | 41 |
| Mult.d F4 F2 F20 | 7 | 19-33 | - | 35 | 42 |
| Ld F6 0(R2) | 7 | 9 | 18-22 | 23 | 43 |
| Add.d F6 F4 F6 | 7 | 36-39 | - | 40 | 44 |
| Sd F6 0(R2) | 8 | 10 | 45-49 | - | 45-49 |
| Addi R1 R1 -4 | 8 | 9 | - | 10 | 50 |
| Addi R2 R2 -4 | 8 | 10 | - | 11 | 51 |
| Bne R1 R0 -8 | 8 | 11 | - | - | 52 |
| Ld F2 0(R1) | 12 | 13 | 23-27 | 28 | 53 |
| Mult.d F4 F2 F20 | 12 | 29-43 | - | 44 | 54 |
| Ld F6 0(R2) | 17 | 18 | 28-32 | 33 | 55 |
| Add.d F6 F4 F6 | 17 | 45-48 | - | 49 | 56 |
| Sd F6 0(R2) | 22 | 23 | 57-61 | - | 57-61 |
| Addi R1 R1 -4 | 22 | 23 | - | 25 | 62 |
| Addi R2 R2 -4 | 22 | 24 | - | 26 | 63 |
| Bne R1 R0 -8 | 22 | 26 | - | - | 64 |
| Add.d F20 F2 F2 | 28 | 29-32 | - | 34 | 65 |

# 4 What Works and Limitations

## 4.1 What Works

Both benchmarks complete with the expected register and memory values at issue widths 1, 2, 3, and 4. The independent sequence demonstrates multiple issue, and the array loop demonstrates dependency handling, register reuse, memory updates, and branch recovery. No incorrect final value or execution failure was observed in these runs. Single-precision checks also confirm rounding before dependent operations, loads, stores, and forwarding at all four issue widths, including the 16777216 + 1 example.

## 4.2 Known Limitations

Integer values use unbounded Python integers, so fixed-width integer overflow is not modeled. Floating-point values and results use binary32; overflow produces signed infinity, and underflow supports subnormal values and signed zero. Floating-point exception flags and traps are not modeled. Memory accesses are restricted to aligned words at byte addresses 0, 4, ..., 252.